04/08/2025

INITIAL STEP:

✅ OBJECTIVE:

Simulate Verilog code (stepper_control.v) that drives a motor using a sequence of 4 output bits.

STEP 1: Open Vivado & Create a New Project

🪜 Do this:

- Open Vivado

- Click "Create Project"

- Project Name: stepper_sim

- Uncheck "Do not specify sources at this time"

- Click Next

📁 STEP 2: Add Design Source (Verilog Code)

🪜 Do this:

- In “Add Sources” → Choose "Add or Create Design Sources" → Click Next

- Click "Create File" → Select Verilog, name it: stepper_control

- Click OK, then Finish

- Paste this code into the new file stepper_control.v:

- Click Save (Ctrl + S)

module stepper_control(

input wire clk,

input wire rst,

input wire enable,

output reg [3:0] motor_out

);

reg [2:0] state;

always @(posedge clk or posedge rst) begin

if (rst) begin

state <= 0;

motor_out <= 4'b0000;

end else if (enable) begin

case(state)

3'd0: motor_out <= 4'b1000;

3'd1: motor_out <= 4'b1100;

3'd2: motor_out <= 4'b0100;

3'd3: motor_out <= 4'b0110;

3'd4: motor_out <= 4'b0010;

3'd5: motor_out <= 4'b0011;

3'd6: motor_out <= 4'b0001;

3'd7: motor_out <= 4'b1001;

default: motor_out <= 4'b0000;

endcase

state <= state + 1;

end

end

endmodule

🧪 STEP 3: Add Simulation Testbench File

🪜 Do this:

- In the next step "Add Simulation Sources" → Click "Create File"

- Choose Verilog, name it: stepper_tb

- Click OK → Finish

- Paste this code into stepper_tb.v:

- Click Save (Ctrl + S)

`timescale 1ns / 1ps

module stepper_tb;

reg clk;

reg rst;

reg enable;

wire [3:0] motor_out;

stepper_control uut (

.clk(clk),

.rst(rst),

.enable(enable),

.motor_out(motor_out)

);

// Clock: 20 ns period (50 MHz)

always #10 clk = ~clk;

initial begin

clk = 0;

rst = 1;

enable = 0;

#50;

rst = 0;

enable = 1;

#500;

$stop;

end

endmodule

STEP 4: Skip Adding Constraints

Just click Next > Finish Project Setup

Since we are only simulating, we don't need constraints for physical pins.

▶️ STEP 5: Run Behavioral Simulation

🪜 Do this:

- On the left sidebar, find "Flow Navigator"

- Under Simulation, click "Run Simulation" > "Run Behavioral Simulation"

Vivado will process for a few seconds and open a waveform window.

📈 STEP 6: View the Output

Look in the waveform:

- You’ll see signals like clk, rst, enable, and motor_out[3:0]

🧹 STEP 7: Close Simulation

When you’re done:

- Click File > Close Simulation

- Or use the command: $finish inside the testbench if you edit it later